ArduinoBoy

A “Modern” Retro Game Console

ДОКУМЕНТАЦИЯ

ИЗГОТВИЛИ:

Божидар Андонов

Петко Люцканов

СЪДЪРЖАНИЕ

Списък от компоненти	3

Описание	4

Електрическа схема	6

Блок схема	7

Описание на функционалността и сорс код	8

Заключение	18

# Списък от компоненти

1. Arduino Uno

2. Bluetooth HC-05 модул

3. MAX7219 модули за контролиране на LED матриците(x2)

Фигура 4- MAX7219 модул, контролиращ LED матриците4

Фигура 3- LED матрицата, чрез която визуално се представят игрите

Фигура 2- Bluetooth HC-05 модул, предаваш информация към и от микроконтролера

Фигура 1- Arduino Uno, микроконтрелора, който извършва всички операции

4. LED матрици 8x8(x2)

# Описание

Нашият проект се казва ArduinoBoy и представлява мини игрова конзола с някои класически игри, а именно - Змията, Тетрис и Понг. Използваме Arduino Uno, за управление на целия проект. Изобразяването на игрите става чрез 2x 8x8 LED матрици, контролирани от MAX7219 модули.

Фигура 5-  Екран за избиране на игра

Специалното на тази конзола е, че няма никакви физически бутони, а комуникацията между потребителя и вградената система се извършва чрез HC-05 Bluetooth модула, който трябва да се сдвои и свърже към смартфон с инсталирано специално разработено(с MitAppInventor) приложение, наречено ArduinoBoyController  за този проект. Потребителят след това има право да избере една от трите игри и да я играе, като тя ще бъде изобразена върху LED матриците.

Фигура 6- Екран на контролера

Контролерът представлява 4 бутона във всяка посока, както и A и B бутони, които имитират оформлението на GameBoy конзолата, която всъщност и е вдъхновението за този проект.

Тетрисът представлява класически тетрис, т.е без гравитация и без наместваща ротация – точно както в класиката. Целта на играта е да не се стигне най-горният ред, защото нова фигурка няма да може да бъде поставена. Играчът може да движи и да върти тетроминотата, както по часовниковата стрелка, така и обратно. Колкото повече фигурки бъдат сложени, толкова по-бърза и трудна става играта.

Змията също е класическа ретро игра, която решихме да включим в проекта. Тя представлява змия, която се опитва да събере възможно най-много храна без да се “ухапе”(да се блъсне в себе си). Играчът може да контролира накъде се движи змията с четирите бутона за посока. В тази версия няма стени и змията продължава от другата страна на матрицата ако премине през ръба й. Тук също колкото повече храна събираш, толкова по-бързо се движи самата змия.

Фигура 9- Игра на понг

Фигура 8- Игра на змия

Фигура 7- Игра на тетрис

Понг играта представлява топче, което се подава от двама играчи, като всеки може да се движи само хоризонтално. Ако даден играч изпусне топчето, другият печели точка. Първият, достигнал 10 точки, побеждава. В ArduinoBoy версията на играта, играе само един играч, а A.I. представлява другия играч. За да са по-интересни(и по-забързани) рундовете, колкото повече пъти топчето бива ударено от даден играч, толкова неговата скорост нараства.

# Електрическа схема

Фигура 10 - Електрическа схема

Това е електрическата схема на проекта ни, която се състои от компонентите включени горе. MAX7219 модулите изглеждат различно, но това е само визуално. Всички пинове са свързани аналогично на физическия проект.

# Блок схема

Фигура 11- Блок схема

Това е блок схемата на проекта ни. Батерията подава захранване на проекта. Телефонът се свързва към Arduino-то чрез BlueTooth модула и комуникацията йм протича през него. Когато потребителят прати някаква команда, Arduino-то изпраща информация към MAX7219 модулите, които от своя страна включват и изключват определени LED-ове на матриците.

# Описание на функционалността и сорс код

Това е целият код използван за програмиране на Arduino Uno-то. Чрез коментарите и допълнително вмъкнатия текст ще разбререте функционалността. Първо инициализираме основните променливи, които ще използваме по време на изпълнение на приложението. Определяме и sprite-ове, които предефинират какво да се покаже на дисплея когато бъдат извикани.

#include <ArduinoSTL.h>

#include <LedControl.h>

#include <vector>

#define DINPin 11  // DataIn pin for the MAX7219 module

#define CSPin 12   // Load pin for the MAX7219 module

#define CLKPin 13  // Clock pin for the MAX7219 module

// The following are all used structs within the project

// Location is used to map certain game elements to the LED matrix

struct Location

{

byte x; // [0-15] the row index of the element

byte y; // [0-7] the column index of the element

};

// This struct is specifically made for the tetris game and is used to

// track where the tetromino is.

struct Tetromino

{

// All of the blocks represent one location element, so that the tetromino

// can be displayed properly

Location block1;

Location block2;

Location block3;

Location block4;

Location center; // The center is used to determine the rotation point of the tetromino

};

// This struct is used to locate both the A.I. paddle and the player controlled one in the Pong game

struct Paddle

{

// It consists of two Location objects as well, they determine where the paddle is located

Location block1;

Location block2;

};

// Direction is a struct, used in the pong game and its purpose is to determine what

// direction the ball will move in.

struct Direction

{

short x; // [-1...1] the row direction, -1 is upwards, 1 is downwards

short y; // [-1...1] the column direction, -1 is left, 1 is right

};

// All of the following are sprites, which are used to display generic information

// on the LED displays when an event is fired. All of them are in binary, represents

// whether the light is on or off.

// The sprite used in the game over screen.

byte gameOverSprite[16] =

{

B11110110,

B10001001,

B10111111,

B01100100,

…

…

Има много от тези спрайтове, затова ще ги пропуснем

…

B00000000,

B00000000

};

// The following variables are used by all of the games

LedControl matrixController(DINPin, CLKPin, CSPin, 2); // The controller for the LED matrix.

unsigned long timer; // timer, which uses the millis() function to determine when certain events should be fired.

bool isGameOver; // Variable, which determines whether the current game is over.

int playerScore; // Variable, used to store the player's current score.

// The following variables are used by the Tetris game

Tetromino tetromino; // The tetromino that the user currently has control of

bool tetrisMatrix[16][8]; // The matrix, used to determine which LEDs turn on

short fallingTetrominoDelay; // The delay used to determine how long it will be until the tetromino goes down one step.

// The following variables are used by the Snake game

char prevDirection; // The previous direction the snake followed.

char direction; // The current direction the snake is following

Location food; // The row and column coordinates of the food.

bool foodState; // Determines what state the blinking food is in (true -> LED on, false -> LED off)

short const scorePerFood = 10; // A constant used to determine how many points each eaten food gives.

След декларирането на променливите, декларираме основните методи, с които работи ардуиното – setup() и loop(), в които нулираме основни променливи и чакаме потребителя да избере игра. Метод playSnake() e основният метод, контролиращ играта “Snake”, а setupSnake() е този, който я подготвя  преди всеки неин пуск.

short snakeMovementDelay; // The delay used to determine how long it will be until the snake follows the current direction again

std::vector<Location> snake; // A vector, containing all of the coordinates of the snake.

// Pong following variables are used by the Pong game

Paddle playerPaddle; // The paddle controlled by the player

Paddle aiPaddle; // The paddle controlled by the A.I.

Location ball; // The current location of the ball.

Direction ballDirection; // The direction in which the ball is going in

byte const pointsToWin = 10; // A constant, used to determine how many points each side needs to win the Pong game

short const aiMovementDelay = 220; // The constraint, which makes it impossible for the A.I. to win every game.

short ballDelay; // The delay used to determine how long it will be until the ball changes its position

unsigned long aiTimer; // An additional timer, whose purpose is to determine whether the A.I. can move its paddle yet.

int numberOfHits; // Number of times the ball was hit by either of the players. Each time it is hit, the velocity of the ball increases.

int aiScore; // An additional score counter, used for the A.I.

void setup()

{

// Open the serial port for communication

Serial.begin(9600);

// Wake the LED boards up

matrixController.shutdown(0, false);

matrixController.shutdown(1, false);

// Set the intensity of the display

matrixController.setIntensity(0, 0);

matrixController.setIntensity(1, 0);

// Clear the display

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

// Giving a random seed, so that the games spawn random items in random locations. Uses

// the noise the A0 port

randomSeed(analogRead(A0));

}

void loop()

{

// Wait for a signal from the smartphone app. Once a game is selected

// on the phone, the function responsible for the game is called.

if (Serial.available() >= 0)

{

char requestedGame = Serial.read();

if (requestedGame == 'T')

{

playTetris();

}

else if (requestedGame == 'S')

{

playSnake();

}

else if (requestedGame == 'P')

{

playPong();

}

}

}

// The following methods are used to play the SNAKE game.

// This method is the main one, responsible for running the SNAKE game.

void playSnake()

{

setupSnake();

// Start the timer

timer = millis();

while (!isGameOver)

{

// When information is sent through the mobile application, it is interpreted here.

if (Serial.available() >= 0)

{

char input = Serial.read();

// 'L' - Left; 'R' - Right; 'D' - Down; 'U' - Up

if (input == 'L' || input == 'R' || input == 'D' || input == 'U')

{

char prevDirection = direction;

char newDirection =  input;

direction = changeSnakeDirection(prevDirection, newDirection);

}

}

// When a certain time passes, move the snake in the selected direction

if (millis() - timer >= snakeMovementDelay)

{

timer = millis();

moveSnake();

}

}

displayGameOverScreen();

}

// This method is used to setup the Snake game every time before it is run.

void setupSnake()

{

//Setting variables to their default starting values

isGameOver = false;

playerScore = 0;

foodState = true;

snakeMovementDelay = 800;

// If anything from the snake in the last game remained, it's cleared.

while (!snake.empty())

{

snake.pop_back();

}

// Set the starting location for the snake

Location startingBlock;

startingBlock.x = 7;

startingBlock.y = 4;

snake.push_back(startingBlock);

startingBlock.x = 7;

startingBlock.y = 3;

snake.push_back(startingBlock);

startingBlock.x = 7;

startingBlock.y = 2;

snake.push_back(startingBlock);

// Get a new food location and then display the game on the LED matrices

newFood();

printSnakeGameBoard();

}

Тук можем да видим методите за смяна на посоката и как се генерира нова храна. Също така декларираме методи за показване на състоянието на игралното поле и змията. Започнат е и методът, който придвижва змията в 2D пространството.

// This method is used to change the direction of the snake

char changeSnakeDirection(char prevDirection, char newDirection)

{

// If the previous direction is exactly the opposite to the requested one,

// ex. Left and Right, then don't change it (The snake can't turn 180 degrees)

switch (newDirection)

{

case 'R':

if (prevDirection == 'L')

{

return 'L';

}

else return 'R';

break;

case 'L':

if (prevDirection == 'R')

{

return 'R';

}

else return 'L';

break;

case 'D':

if (prevDirection == 'U')

{

return 'U';

}

else return 'D';

break;

case 'U':

if (prevDirection == 'D')

{

return 'D';

}

else return 'U';

break;

}

}

// This method is used to generate a new food when the previous one has been collected by the snake.

void newFood()

{

bool needNewLocation = true; // Used to determine whether the generated location overlaps with the snake.

byte x, y; // The coordinates for the new food.

while (needNewLocation)

{

x = random(0, 15);

y = random(0, 7);

// Checking whether the generated location overlaps with the snake

for (std::vector<Location>::iterator i = snake.begin(); i != snake.end(); i++)

{

if (i -> x == x && i -> y == y)

{

needNewLocation = true;

break;

}

else needNewLocation = false;

}

if (!needNewLocation)

{

food.x = x;

food.y = y;

}

}

// Display the new food alongside the snake

printSnakeGameBoard();

}

// This method is used to display the Snake game elements

void printSnakeGameBoard()

{

// Clear the displays

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

// Determine what state of the LED of the blinking food must be in - On or off

foodState = !foodState;

matrixController.setLed(food.x / 8, food.x % 8, food.y, foodState);

// Light up the snake

for (std::vector<Location>::iterator i = snake.begin(); i != snake.end(); i++)

{

matrixController.setLed(i -> x / 8, i -> x % 8, i -> y, true);

}

}

// This method is used to turn off all LEDs associated with the snake itself

void turnOffOldSnakeLocation()

{

for (std::vector<Location>::iterator i = snake.begin() + 1; i != snake.end(); i++)

{

matrixController.setLed(i -> x / 8, i -> x % 8, i -> y, false);

}

}

// This method is responsible for making the snake move

void moveSnake()

{

turnOffOldSnakeLocation();

// Get the back and the front of the snake

Location backLocation = snake.back();

snake.pop_back();

Location frontLocation = snake.front();

// Get the new location of the front of the snake, dependent on the

// direction it moves in. If it goes outside the matrix, it will return

// from the other side.

Location newFrontLocation;

switch (direction)

{

case 'R':

{

newFrontLocation.x = frontLocation.x;

newFrontLocation.y = (frontLocation.y + 1) % 8;

break;

}

case 'L':

{

newFrontLocation.x = frontLocation.x;

if (frontLocation.y == 0)

{

newFrontLocation.y = 7;

}

else newFrontLocation.y = frontLocation.y - 1;

break;

}

case 'D':

{

newFrontLocation.x = (frontLocation.x + 1) % 16;

newFrontLocation.y = frontLocation.y;

break;

}

case 'U':

{

if (frontLocation.x == 0)

{

Трябва да обърнем внимание на setNewSnakeSpeed() метода. Той се състои в това, че колкото по-голяма става змията, толкова по-бързо започва да се придвижва. Виждаме и основните методи за игрането на тетрис (playTetris() и setupTetris()), които са аналогични на тези на змията.

newFrontLocation.x = 15;

}

else newFrontLocation.x = frontLocation.x - 1;

newFrontLocation.y = frontLocation.y;

break;

}

}

// Insert the new front location into the snake vector and collect food if the

// snake overlaps with it

snake.insert(snake.begin(), newFrontLocation);

collectFood(backLocation);

printSnakeGameBoard();

checkIfSnakeGameOver();

}

// This method is responsible for enlarging the snake when food is collected

// and adds points to the score counter for doing so.

void collectFood(Location backLocation)

{

if (snake[0].x == food.x && snake[0].y == food.y)

{

snake.push_back(backLocation);

addSnakeScore();

newFood();

}

}

// This method is used to add points to the score counter and send that

// information to the mobile phone, which on its turn displays it.

void addSnakeScore()

{

playerScore += scorePerFood;

String stringToPrint = "C" + String(playerScore);

Serial.print(stringToPrint);

setNewSnakeSpeed();

}

// This method manages how fast the snake is moving. The more one progresses,

// the faster the snake becomes.

void setNewSnakeSpeed()

{

if (snakeMovementDelay > 300)

{

snakeMovementDelay -= 10;

}

}

// This method is responsible for determining whether it is game over.

void checkIfSnakeGameOver()

{

// Determine whether the front of the snake collides with a part of its body.

// If that's the case, then it is game over and the game stops.

Location front = snake.front();

byte count = 0;

for (std::vector<Location>::iterator i = snake.begin(); i != snake.end(); i++)

{

if (front.x == i -> x && front.y == i -> y)

{

count++;

}

}

if (count > 1)

{

isGameOver = true;

}

}

// The following methods are used for playing the TETRIS game

// This is the main method, used to play the tetris game.

void playTetris()

{

setupTetris();

// Start the timer

timer = millis();

while (!isGameOver)

{

// This is triggered when it is time for the tetromino to go down one step

if (millis() - timer >= fallingTetrominoDelay)

{

timer = millis();

shiftDown();

String stringToPrint = "C" + String(playerScore);

Serial.print(stringToPrint);

}

// Managing user input.

if (Serial.available() > 0)

{

char command = Serial.read();

if (command == 'L')

{

shiftLeft();

}

else if (command == 'R')

{

shiftRight();

}

else if (command == 'A')

{

rotate(-1, 1);

}

else if (command == 'B')

{

rotate(1, -1);

}

else if (command == 'D')

{

fastForward();

}

}

}

displayGameOverScreen();

}

// This method is used to setup the tetris game

void setupTetris()

{

// Assign default values to the variables.

playerScore = 0;

fallingTetrominoDelay = 1000;

isGameOver = false;

// Clear the tetris matrix if something remains from the last game and

// get the first tetromino

for (int i = 0; i < 16; i++)

{

for (int j = 0; j < 8; j++)

{

tetrisMatrix[i][j] = false;

}

}

getNewTetromino();

}

// This method is responsible for fast-forwarding the placement

Тук са методите за по-бързото падане на тетроминото, както и алгоритъма за неговото обръщане в зависимост от това, дали потребителя иска да го върти по часовниковата стрелка или срещу нея

// of the tetromino when the Down button has been pressed

void fastForward()

{

while (tetrisMatrix[tetromino.block1.x + 1][tetromino.block1.y] != 1 &&

tetrisMatrix[tetromino.block2.x + 1][tetromino.block2.y] != 1 &&

tetrisMatrix[tetromino.block3.x + 1][tetromino.block3.y] != 1 &&

tetrisMatrix[tetromino.block4.x + 1][tetromino.block4.y] != 1 &&

tetromino.block1.x != 15 && tetromino.block2.x != 15 &&

tetromino.block3.x != 15 && tetromino.block4.x != 15)

{

shiftDown();

delay(fallingTetrominoDelay / 7);

}

}

// This method is used to rotate the tetromino in the best way possible

// in a clockwise or anti-clockwise direction.

void rotate(int rotationXIndex, int rotationYIndex)

{

// The center is zero only when it is the 'O' tetromino

if (tetromino.center.x == 0 && tetromino.center.y == 0)

{

return;

}

turnOffOldTetrominoLocation();

// The numbers on those variables correspond to the Tetromino struct ones.

// This algorithm is used to determine where the new blocks will be located

// after the rotation.

Location tempPosition1, tempPosition2, tempPosition3, tempPosition4;

Location relativePosition1, relativePosition2, relativePosition3, relativePosition4;

Location newLocation1, newLocation2, newLocation3, newLocation4;

tempPosition1.x = tetromino.block1.x - tetromino.center.x;

tempPosition1.y = tetromino.block1.y - tetromino.center.y;

relativePosition1.x = rotationXIndex * tempPosition1.y;

relativePosition1.y = rotationYIndex * tempPosition1.x;

newLocation1.x = tetromino.center.x + relativePosition1.x;

newLocation1.y = tetromino.center.y + relativePosition1.y;

…

Тук, всичко също е аналогично и го пропускаме

…

…

newLocation4.x = tetromino.center.x + relativePosition4.x;

newLocation4.y = tetromino.center.y + relativePosition4.y;

// When the new location is overlapping with an already placed

// tetromino or is outside the bounds of the screen, the rotation

// is not completed.

if (newLocation1.x < 0 || newLocation1.x > 15 ||

newLocation1.y < 0 || newLocation1.y > 7 ||

newLocation2.x < 0 || newLocation2.x > 15 ||

newLocation2.y < 0 || newLocation2.y > 7 ||

newLocation3.x < 0 || newLocation3.x > 15 ||

newLocation3.y < 0 || newLocation3.y > 7 ||

newLocation4.x < 0 || newLocation4.x > 15 ||

newLocation4.y < 0 || newLocation4.y > 7)

{

turnOnNewTetrominoLocation();

return;

}

else if (tetrisMatrix[newLocation1.x][newLocation1.y] == false &&

tetrisMatrix[newLocation2.x][newLocation2.y] == false &&

tetrisMatrix[newLocation3.x][newLocation3.y] == false &&

tetrisMatrix[newLocation4.x][newLocation4.y] == false)

{

tetromino.block1 = newLocation1;

tetromino.block2 = newLocation2;

tetromino.block3 = newLocation3;

tetromino.block4 = newLocation4;

}

turnOnNewTetrominoLocation();

}

Метод за придвижване надолу.

// This method is used to bring the tetromino one step down

void shiftDown()

{

turnOffOldTetrominoLocation();

// Check whether the tetromino is already at the bottom or whether it

// is touching the matrix

if (tetromino.block1.x == 15 || tetromino.block2.x == 15 || tetromino.block3.x == 15 || tetromino.block4.x == 15)

{

tetrisMatrix[tetromino.block1.x][tetromino.block1.y] = 1;

tetrisMatrix[tetromino.block2.x][tetromino.block2.y] = 1;

tetrisMatrix[tetromino.block3.x][tetromino.block3.y] = 1;

tetrisMatrix[tetromino.block4.x][tetromino.block4.y] = 1;

addTetrisScore(5);

getNewTetromino();

return;

}

else if (tetrisMatrix[tetromino.block1.x + 1][tetromino.block1.y] == 1 ||

tetrisMatrix[tetromino.block2.x + 1][tetromino.block2.y] == 1 ||

tetrisMatrix[tetromino.block3.x + 1][tetromino.block3.y] == 1 ||

Тук са декларирани методите за преместване на тетроминото наляво и надясно, както и за генериране на ново такова, когато е докоснало някоя друга фигура. Можем да видим и проверка за край на играта.

tetrisMatrix [tetromino.block4.x + 1][tetromino.block4.y] == 1)

{

tetrisMatrix[tetromino.block1.x][tetromino.block1.y] = 1;

tetrisMatrix[tetromino.block2.x][tetromino.block2.y] = 1;

tetrisMatrix[tetromino.block3.x][tetromino.block3.y] = 1;

tetrisMatrix[tetromino.block4.x][tetromino.block4.y] = 1;

addTetrisScore(5);

getNewTetromino();

return;

}

// Offset the tetromino by one to the bottom.

tetromino.block1.x++;

tetromino.block2.x++;

tetromino.block3.x++;

tetromino.block4.x++;

tetromino.center.x++;

turnOnNewTetrominoLocation();

}

// This method is used to shift the tetromino left when the user requests it

void shiftLeft()

{

// Check whether it is already at the leftmost position or the space is already occupied

if (tetromino.block1.y == 0 || tetromino.block2.y == 0 || tetromino.block3.y == 0 || tetromino.block4.y == 0)

{

return;

}

else if (tetrisMatrix[tetromino.block1.x][tetromino.block1.y - 1] == 1 ||

tetrisMatrix[tetromino.block2.x][tetromino.block2.y - 1] == 1 ||

tetrisMatrix[tetromino.block3.x][tetromino.block3.y - 1] == 1 ||

tetrisMatrix[tetromino.block4.x][tetromino.block4.y - 1] == 1)

{

return;

}

// Offset the tetromino by one to the left.

turnOffOldTetrominoLocation();

tetromino.block1.y--;

tetromino.block2.y--;

tetromino.block3.y--;

tetromino.block4.y--;

tetromino.center.y--;

turnOnNewTetrominoLocation();

}

// This method is used to shift the tetromino right when the user requests it

void shiftRight()

{

// Check whether it is already at the rightmost position or the space is already occupied

if (tetromino.block1.y == 7 || tetromino.block2.y == 7 || tetromino.block3.y == 7 || tetromino.block4.y == 7)

{

return;

}

else if (tetrisMatrix[tetromino.block1.x][tetromino.block1.y + 1] == 1 ||

tetrisMatrix[tetromino.block2.x][tetromino.block2.y + 1] == 1 ||

tetrisMatrix[tetromino.block3.x][tetromino.block3.y + 1] == 1 ||

tetrisMatrix[tetromino.block4.x][tetromino.block4.y + 1] == 1)

{

return;

}

// Offset the tetromino by one to the right.

turnOffOldTetrominoLocation();

tetromino.block1.y++;

tetromino.block2.y++;

tetromino.block3.y++;

tetromino.block4.y++;

tetromino.center.y++;

turnOnNewTetrominoLocation();

}

// This method is used to generate a new tetromino when the old one has been placed.

void getNewTetromino()

{

clearFullRows();

int tetrominoIndex = random(0, 7);

switch (tetrominoIndex)

{

// Setting all of the locations manually, depending on the chosen tetromino.

case 0:

tetromino.block1.x = 0;  //  - - o o o o - -

tetromino.block1.y = 2;

tetromino.block2.x = 0;

tetromino.block2.y = 3;

tetromino.block3.x = 0;

tetromino.block3.y = 4;

tetromino.block4.x = 0;

tetromino.block4.y = 5;

tetromino.center.x = 0;

tetromino.center.y = 3;

break;

case 1:

tetromino.block1.x = 0;  //  - - o - - - - -

tetromino.block1.y = 2;  //  - - o o o - - -

tetromino.block2.x = 1;

…

…

Всички генерирания са сходни така че ще ги пропуснем.

…

tetromino.block3.y = 3;

tetromino.block4.x = 1;

tetromino.block4.y = 4;

tetromino.center.x = 1;

tetromino.center.y = 3;

break;

}

turnOnNewTetrominoLocation();

checkIfTetrisGameOver();

}

// This method determines whether it is game over in the game of Tetris

void checkIfTetrisGameOver()

{

// If a tetromino overlaps with a location from the matrix, then there is no space

Методите тук отговарят за изчистване на редовете когато се напълнят, както и за кои части от LED матрицата трябва да светнат. Тук отново виждаме, че колкото повече точки се трупат, толково по-бързо падат тетроминотата.

// and that means the game is over.

if (tetrisMatrix[tetromino.block1.x][tetromino.block1.y] == true ||

tetrisMatrix[tetromino.block2.x][tetromino.block2.y] == true ||

tetrisMatrix[tetromino.block3.x][tetromino.block3.y] == true ||

tetrisMatrix[tetromino.block4.x][tetromino.block4.y] == true)

{

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

isGameOver = true;

}

}

// This method is responsible for clearing any full rows that have

// formed due to placing a tetromino in a certain location.

void clearFullRows()

{

for (int i = 0; i < 16; i++)

{

// Find out whether the current row (i) is full.

bool isRowFull = true;

for (int j = 0; j < 8; j++)

{

if (tetrisMatrix[i][j] == false)

{

isRowFull = false;

break;

}

}

// If it is, clear it and move everything above it one step lower.

if (isRowFull)

{

for (int j = 0; j < 8 ; j++)

{

tetrisMatrix[i][j] = false;

}

lightUpTetrisMatrix();

delay(200);

for (int k = i; k > 0 ; k--)

{

for (int l = 0; l < 8; l++)

{

tetrisMatrix[k][l] = tetrisMatrix[k - 1][l];

tetrisMatrix[k - 1][l] = false;

}

}

lightUpTetrisMatrix();

// Add score for clearing the line.

addTetrisScore(15);

i--;

}

}

}

// This method adds points to the score counter

void addTetrisScore(short scoreToAdd)

{

playerScore += scoreToAdd;

setTetrisSpeed();

}

// This method sets the speed at which the tetromino falls. The longer

// the game goes on, the faster this speed is and the harder the game becomes.

void setTetrisSpeed()

{

if (fallingTetrominoDelay > 300)

{

fallingTetrominoDelay = 1000 - playerScore;

}

}

// This method turns off the old tetromino location.

void turnOffOldTetrominoLocation()

{

// Refresh the whole matrix first.

lightUpTetrisMatrix();

Location block = tetromino.block1;

matrixController.setLed(block.x / 8, block.x % 8, block.y, false);

block = tetromino.block2;

matrixController.setLed(block.x / 8, block.x % 8, block.y, false);

block = tetromino.block3;

matrixController.setLed(block.x / 8, block.x % 8, block.y, false);

block = tetromino.block4;

matrixController.setLed(block.x / 8, block.x % 8, block.y, false);

}

// This method lights up the tetromino on the LED matrices.

void turnOnNewTetrominoLocation()

{

// Refresh the whole matrix first.

lightUpTetrisMatrix();

Location block = tetromino.block1;

matrixController.setLed(block.x / 8, block.x % 8, block.y, true);

block = tetromino.block2;

matrixController.setLed(block.x / 8, block.x % 8, block.y, true);

block = tetromino.block3;

matrixController.setLed(block.x / 8, block.x % 8, block.y, true);

block = tetromino.block4;

matrixController.setLed(block.x / 8, block.x % 8, block.y, true);

}

// This method is used to light up the LEDs according to the tetrisMatrix array.

void lightUpTetrisMatrix()

{

for (int i = 0; i < 16; i++)

{

for (int j = 0; j < 8; j++)

{

matrixController.setLed(i / 8, i % 8, j, tetrisMatrix[i][j]);

}

}

}

// This method is responsible for displaying the game over screen on both the

// snake and tetris games.

void displayGameOverScreen()

{

for (int i = 0; i < 16; i++)

{

matrixController.setRow(i / 8, i % 8, gameOverSprite[i]);

}

while (Serial.available() == 0) {}

Serial.write('V');

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

}

// The following methods are used for playing the PONG game

Всичко оттук нататък е за играта Понг. Отново виждаме двата основни метода за стартиране на играта, както и част от метода за движение на топчето по игралното поле.

// This method is the main one, used to run the Pong game properly.

void playPong()

{

// Setup the game and start the timers.

setupPong();

timer = millis();

aiTimer = millis();

while (!isGameOver)

{

// When the ball needs to move, the method responsible for that is called.

if (millis() - timer > ballDelay)

{

moveBall();

timer = millis();

}

// If the A.I. is allowed to make a move, it does so

if (millis() - aiTimer > aiMovementDelay)

{

movePongAi();

aiTimer = millis();

}

// Interpret information sent by the player's smartphone;

if (Serial.available() > 0)

{

char dir = Serial.read();

if (dir == 'R' || dir == 'L')

{

movePongPlayer(dir);

}

}

}

displayPongWinner();

}

// This method is used to setup the Pong game

void setupPong()

{

// Set variables to their default values.

String scoreToSend = "C0:0";

Serial.print(scoreToSend);

isGameOver = false;

playerScore = 0;

aiScore = 0;

// Call a method to begin a new round.

newPongRound();

}

// This method is responsible for creating new rounds after a point has

// been scored.

void newPongRound()

{

// Set variables to their default values

ballDelay = 250;

numberOfHits = 0;

ball.x = random(1, 8);

ball.y = random(1, 7);

ballDirection.y = random(-1, 2);

ballDirection.x = 1;

playerPaddle.block1.x = 15;

playerPaddle.block1.y = 3;

playerPaddle.block2.x = 15;

playerPaddle.block2.y = 4;

aiPaddle.block1.x = 0;

aiPaddle.block1.y = 3;

aiPaddle.block2.x = 0;

aiPaddle.block2.y = 4;

// Light up the proper LEDs on the display.

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

matrixController.setLed(0, 0, aiPaddle.block1.y, true);

matrixController.setLed(0, 0, aiPaddle.block2.y, true);

matrixController.setLed(1, 7, playerPaddle.block1.y, true);

matrixController.setLed(1, 7, playerPaddle.block2.y, true);

matrixController.setLed(ball.x / 8, ball.x % 8, ball.y, true);

// Wait one second so the player won't be caught off-guard.

delay(1000);

}

// This method is responsible for moving the ball in a proper direction

void moveBall()

{

// If Y is already set, we won't be able to change it at a later point.

bool isYSet = false;

// Emulate bouncing off the walls.

if (ball.y == 0)

{

if (ballDirection.y == -1)

{

ballDirection.y = 1;

isYSet = true;

}

}

else if (ball.y == 7)

{

if (ballDirection.y == 1)

{

ballDirection.y = -1;

isYSet = true;

}

}

// Emulate hitting the paddle or falling below it.

if (ball.x == 1 && ballDirection.x == -1)

{

// When the ball is about to hit the paddle, do this:

if (aiPaddle.block1.y == ball.y + ballDirection.y || aiPaddle.block2.y == ball.y + ballDirection.y)

{

ballDirection.x = 1;

numberOfHits++;

if (!isYSet)

{

ballDirection.y = random(-1, 2);

}

setBallSpeed();

}

// else the other player gets a point.

else

{

playerScore++;

checkIfPongGameOver();

return;

}

}

else if (ball.x == 14 && ballDirection.x == 1)

{

if (playerPaddle.block1.y == ball.y + ballDirection.y || playerPaddle.block2.y == ball.y + ballDirection.y)

{

ballDirection.x = -1;

numberOfHits++;

if (!isYSet)

{

ballDirection.y = random(-1, 2);

}

setBallSpeed();

Тук, трябва да обърнем внимание на алгоритъма, с който A.I. се придвижва, както и на този, с който играчът се мести.

}

else

{

aiScore++;

checkIfPongGameOver();

return;

}

}

// Turning the old ball location off, setting the new location, based on the

// direction it has to go and lighting up the new location.

matrixController.setLed(ball.x / 8, ball.x % 8, ball.y, false);

ball.x += ballDirection.x;

ball.y += ballDirection.y;

matrixController.setLed(ball.x / 8, ball.x % 8, ball.y, true);

}

// This method is used to determine whether it is game over, if it is

// not yet, it displayes the current scores.

void checkIfPongGameOver()

{

if (playerScore == pointsToWin || aiScore == pointsToWin)

{

isGameOver = true;

return;

}

else

{

displayScore();

}

}

// This method is used to set the ball velocity. The more times the ball is hit,

// the faster it becomes.

void setBallSpeed()

{

if (ballDelay >= 100)

{

ballDelay = 250 - numberOfHits * 10;

}

}

// This method determines what direction the A.I. will move in and whether

// it will move at all.

void movePongAi()

{

// The A.I. can only move if the ball is heading towards it.

if (ballDirection.x == -1)

{

// Specific scenarios for the A.I.

if (ball.y == 0 && ball.x == 1 && aiPaddle.block1.y == 0)

{

return;

}

if (ball.y == 7 && ball.x == 1 && aiPaddle.block2.y == 7)

{

return;

}

// Turning off the old location of the A.I. paddle.

matrixController.setLed(0, 0, aiPaddle.block1.y, false);

matrixController.setLed(0, 0, aiPaddle.block2.y, false);

// Determine what direction the paddle should go in, based on several factors.

if (ballDirection.y == 0)

{

if (ball.y < aiPaddle.block1.y)

{

aiPaddle.block1.y--;

aiPaddle.block2.y--;

}

else if (ball.y > aiPaddle.block2.y)

{

aiPaddle.block1.y++;

aiPaddle.block2.y++;

}

}

else if (ballDirection.y == 1)

{

if (ball.y < aiPaddle.block1.y)

{

aiPaddle.block1.y--;

aiPaddle.block2.y--;

}

else if (ball.y > aiPaddle.block1.y && aiPaddle.block2.y < 7)

{

aiPaddle.block1.y++;

aiPaddle.block2.y++;

}

}

else

{

if (ball.y < aiPaddle.block2.y && aiPaddle.block1.y > 0)

{

aiPaddle.block1.y--;

aiPaddle.block2.y--;

}

else if (ball.y > aiPaddle.block2.y)

{

aiPaddle.block1.y++;

aiPaddle.block2.y++;

}

}

// Light up the new A.I. paddle location.

matrixController.setLed(0, 0, aiPaddle.block1.y, true);

matrixController.setLed(0, 0, aiPaddle.block2.y, true);

}

}

// This method is used to change the position of the player paddle

void movePongPlayer(char dir)

{

// Clear the old paddle locations.

matrixController.setLed(1, 7, playerPaddle.block1.y, false);

matrixController.setLed(1, 7, playerPaddle.block2.y, false);

// Depending on the direction the paddle needs to be moved in, different actions occur.

if (dir == 'R')

{

if (playerPaddle.block2.y < 7)

{

playerPaddle.block1.y++;

playerPaddle.block2.y++;

}

}

else if (dir == 'L')

{

if (playerPaddle.block1.y > 0)

{

playerPaddle.block1.y--;

playerPaddle.block2.y--;

}

}

// Light up the new paddle location.

matrixController.setLed(1, 7, playerPaddle.block1.y, true);

Метод displayScore() ползваме за да покажем резултатите между рундовете на Понг, както и дали играчът е победил или загубил.

matrixController.setLed(1, 7, playerPaddle.block2.y, true);

}

// This method is used to display the score after either of the two

// players scores a point.

void displayScore()

{

// Clear the displays

matrixController.clearDisplay(0);

matrixController.clearDisplay(1);

// Send information, regarding the scores to the mobile phone

String scoresToSend = "C" + String(playerScore) + ":" + String(aiScore);

Serial.print(scoresToSend);

// Display the corresponding numbers on the LED matrices.

for (int i = 0; i < 8; i++)

{

matrixController.setRow(0, i, numberSprites[aiScore][i]);

matrixController.setRow(1, i, numberSprites[playerScore][i]);

}

// Wait for user input before continuing and begin a new round.

while (Serial.available() == 0) {}

Serial.read();

newPongRound();

}

// This method is used to display the winner, once the Pong game has been finished.

void displayPongWinner()

{

if (playerScore == pointsToWin)

{

displayWin();

}

else if (aiScore == pointsToWin)

{

displayLoss();

}

// Wait for input from the user.

while (Serial.available() == 0) {}

Serial.write('V');

}

// This method uses the win sprite to display the word 'WIN' on the LED matrices.

void displayWin()

{

for (int i = 0; i < 16; i++)

{

matrixController.setRow(i / 8, i % 8, winSprite[i]);

}

}

// This method uses the loss sprite to display the word 'LOSS' on the LED matrices.

void displayLoss()

{

for (int i = 0; i < 16; i++)

{

matrixController.setRow(i / 8, i % 8, lossSprite[i]);

}

}

# Заключение

Създадохме проект, който цели да пресъздаде една ретро конзола с помощта на Arduino, като съчетаем остарялото с модерното. Това чувство наистина се постига от ниската “резолюция” на екрана(16х8) и ползване на Bluetooth технологии и смартфон за управлението й. Името ArduinoBoy все пак е съчетание от ползвания сравнително нов микроконтролер и една остаряла, но класическа конзола, което също придава това усещане.

Доволни сме от постигнатото, но искаме да отбележим, че както всеки проект, така и този има някои недостатъци:

- Ниската резолюция наистина ограничава набора от игри, които могат да бъдат програмирани.

- Потребителят трябва да изтегли отделно приложение за да управлява случващото се на екрана.

- Не на всеки ще му се харесат подбраните игри и този “oldschool” вид на конзолата

- Проектът е обемист и др.

Въпреки тези недостатъци, смятаме, че проектът има и

перспективи за развитие:

- Може да се добави функционалност за спиране на текущата игра и връщане към екрана за избиране на нова такава

- Може да се обогати преживяването, ако се включат и звуци към игрите и др.